Start by building the example launcher, then turn it into your own unique design.

Download documents at teachergeek.com/learn

For use with TeacherGeek Projectile Launcher Activity, 
or Maker Cart available at teachergeek.com.

Here are TeacherGeek components to make the example projectile launcher, 
and then turn it into your own unique design.

| 10 - Blocks | 5 - Dowels 30cm (12″) | 2 - Connector Strips | 1 - Hole Plate |  | 1 - Ping Pong Ball |
| --- | --- | --- | --- | --- | --- |
| 2 - Nuts #10 | 2 - Lock Nuts #10 | 1 - 38mm Screw 38mm (1½″) #10 | 1 - 25mm Screw (1″) #10 |  | 10 - Rubber Bands |
|  |  |  |  | You might need to cut this from a wire roll. |  |
| 2 - Stop Clips | 1 - Slide Stop 76mm (3″) | 1 - Steel Wire 7.5cm (3″) | 1 - Plastic Coated Wire 8cm (3¼″) |  |  |

►

Components available in the TeacherGeek Ping Pong Launcher Activity, 
TeacherGeek Maker Cart, or at teachergeek.com

This isn’t a kit. You’re going to really build (cut, ream, screw) your launcher. 
Here are the tools you’ll need.

| Multi-Cutter SKU 1823-81 | Reamer SKU 1823-87 | Screwdriver SKU 1823-90 | Pliers SKU 1823-86 |
| --- | --- | --- | --- |

Tools available at teachergeek.com

You will need these non-TeacherGeek supplies:

| Tape Masking, Painter’s, Duct; Any kind of tape will work. | Safety Goggles Should be worn during the activity. | Recycling Materials Add to your design | Scissors For cutting out the protractor and recycling materials. |
| --- | --- | --- | --- |

TeacherGeek Protractor

Print on cardstock or thicker paper for a sturdier protractor.

Printable Protractor Download:

http://teachergeek.org/protractor-angle_finder.pdf

| 13cm | 5cm Cut a 13cm (5”) dowel, and a 5cm (2”) dowel. |  | Push, or tap, each dowel from Step 1 into a block as shown. The dowels should stick out a little. 5cm Block 13cm |
| --- | --- | --- | --- |
|  | Push or tap the dowels from Step 2 into the hole plate, as shown. Hole Plate Tap! |  | Rub a crayon, wax, or bar of soap, on two full (30cm) dowels. This helps the dowels slide. 30cm dowel Slide the dowels Launch Pad from Step 4 through the blocks from Step 3. These will be the legs. Legs |
| TeacherGeek Protractor | Print, cut, and tape a protractor to the launcher, as shown. | Use a dowel or pencil to bend the hook. | Poke! 6.5cm 7.5cm Metal Wire Make sure the metal wire points straight down when the protractor is tilted. Bend a 1.5cm hook at the end of a 7.5cm Steel wire (Not the plastic coated wire). Poke it through the protractor at the . Hang it on the paper as shown. |

Launch Pad

Don’t cover the 
cross-hair (circle) 
with tape.

Change the angle of the launch pad 
by sliding the leg dowels up and down.

Read the angle 
on the protractor.

Legs

►

Congratulations! 		
Your Launch Pad is finished!
Now, it’s time to create 
the Launching Mechanism.

Printable Protractor Available at:

http://teachergeek.org/protractor-angle_finder.pdf

|  | Cut a piece of connector strip, four holes long. 4 holes |  | Connect two rubber bands to the cut connector strip, as shown. Rubber Band Loop! Getting it right can be tricky. Take your time! |
| --- | --- | --- | --- |
|  | 8cm Wire Piece Cut, or find, a 8cm (3”) piece of plastic coated wire. Fold it in half and place each end through a connector strip hole. |  | Twist wire ends together to create a strong loop. This will be the launch mechanism. Wire |
| ► | Slide the launch mechanism rubber bands over the Rubber Bands Launch Pad dowels shown. Give it a try… Pull it back, place a ping-pong ball in the launch mechanism, let go and watch it fly. Can you get it to hit the same spot twice? Probably not… Try building a trigger to help out. |  |  |
|  | Place a 38mm (1 ½”) screw through the bottom of the hole plate, as shown. Fasten it with a nut. Tighten! Nut 38mm screw Top View Underside of Plate |  |  |
|  | Place a stop clip on the screw to create a trigger. Trigger Stop Clip Test Your Launcher Out! |  | Lock Nut Place a lock nut on the screw to prevent the stop clip from sliding off. |

A

Pull back the launching mechansism 
and attach it to the trigger (stop clip).

Place a ping-pong ball 
into the launch mechanism.

Turn the trigger (stop clip) to fire!

B

C

Tip: Make sure trigger is lined-up with the center 
of the ball.

Pull!

Caution: Wear safety glasses. 
Never launch at another person.

C

B

=

Projectile

=

Trajectory

The angle of launch affects the projectile’s trajectory (how high + far it launches).

Small˚

Medium˚

Large˚

Try It Out: Slide the legs 
to change the angle of the launcher? How does angle affect the projectile’s trajectory?

Small

Medium

Large

< 30˚

30˚-60˚

> 60˚

Target (Bull’s-Eye Center)

=

The example launcher is just…ok
You can redesign it much better!

Redesign Your Launcher to Be…

Accurate or how close you get to the actual (true) target or goal (bull’s-eye).

Pew!

Pew!

Precise or how constant (repeatable) the results are (landing in the same spot).

Able to Launch Really, Incredibly Far
Power adds distance to your shots. Change how much power

I’m a little wimpy -
my launch mechanism needs more accuracy!

Use recycling bin materials, Teachergeek components & 3D printing for your design!

There are many ways to send a ball flying. Here are a few ideas…

Sling It

Pull back and let it go! This is the most popular launch mechanism. The example launcher uses it. You can design your own “better” version.

Hit It

Hit it like a golf ball or baseball. Get the launch mechanism moving 
before it contacts the ball.

Toss It

Toss it like a catapult, ballista or trebuchet. Note: It can be difficult to change only one independent variable (see below) with this launch mechanism.

Variables: The things that change in an experiment 
(when you launch a projectile and measure where it goes). 
Design and test your launcher so that only the independent and dependent variables change.

Dependent Variable: The variable being tested and measured. 
Only have one dependent variable in your experiment, such as the distance projectile travels.

Independent Variable: The thing you change in the 
experiment, to test how it effects the dependent variable. 
Only have one independent variable in your experiment, such as launch angle.

What are you waiting for? It’s time to start into a lab 
or engineering challenge. It’s going to be a blast!

Visit teachergeek.com/learn to download launcher lab and challenge documents.