Grades

3-5 version available at teachergeek.com/bridges

6-12+

You Are Here

Go Guide

Start here! Build your example bridge, evolve your design, and begin the Distance Challenge!

Optional Labs

Optional Challenges

-Hooke’s Law Lab (Grades 7+)
-Software Analysis (Grades 10+)
-Physics Analysis (Grades 11+)

-Distance Challenge*
-Strength Challenge*

*See Page 6

Choose how you would like to complete this activity.
Download documents & videos at teachergeek.com/bridges

Other Resources

-Strength Testing Guide
-Engineering Notebook
-Design Grid

Learn how structures react to forces by designing, testing, and improving your own unique bridge!

What parts will you need to create your bridge? The list below includes extra parts so you can experiment and develop your own unique designs.

Strips
30cm (12in.)
SKU 1821-31

Half Strips
15cm (6in.)
SKU 1821-31

Colored Wire
SKU 1821-43

NAME

QTY

11

10

1

Bridge Nuts
size #8
SKU 1824-80

33

Bridge Screws
#8 76mm (3in.)
SKU 1824-78

11

Note: These are not the usual #10 nuts and screws used on TeacherGeek projects. They are thinner (#8), so they can slide through strip holes.

PICTURE

If you do not have half strips, cut or snap them from full strips.

Wire can be used, just like strips, as parts of your bridge.

Roll

OTHER MATERIALS

Scissors

Tape
duct tape preferred

2 Markers

Phillips Screwdriver

to tighten the screws that make the bridge joints

Pliers

to hold the nuts while tightening screws.

How do you connect bridge parts?

Spin a nut onto the screw to create a joint.

Slide a screw through two half strips.

Half Strips

Nut

Screw

Joints

Members

Joints are the points on the structure where members connect.

Get ready to test bridge designs!

Abutments

Abutments: The structures that hold up the ends of a bridge.

Create abutments by taping two markers to the top of a table so they are 30cm (12 in.) apart.

Abutments

You can measure the gap with a full strip.

30cm
(12in.)

Time to test some designs!

Abutments: Structures that transfer the force of the load from the bridge to the ground.

Load: The weight carried by a bridge, which pushes/pulls the bridge downward.

Deflection: When a bridge bends as it carries a load.  Even the strongest bridges will deflect.

Try reinforcing the strip with triangles.

Place a full strip on your abutments.  Gently pull the middle down.  Does it flex?  
Is it stiff?

Pull

Gently

The strip is flexible.  It deflects (bends) easily.  Let’s try and make it deflect less.

Is it stiff?

Pull

Gently

It’s stiffer, but the middle deflects. What happens to the tops of the triangles?

How does the deflection change if we turn it upside down?

Pull

Gently

What’s happening to the bottom points of the triangles?

How can you use wire in your design?

Compression: a member is in compression when it is squeezed or pushed inward.

Molecules push back, trying to stay apart.

Tension: a member is in tension when it is 
pulled outward.

Molecules pull on each other, struggling to stay together.

Nope…  The wire is under compression, and wire bends instead of pushing back.

Add wire to reinforce your structure.  Does the wire stiffen it?

Tip

To attach wire,  
wrap it around 
a screw, then 
twist the wire 
around itself.

Strips are strong under both compression and tension.

You created a truss (half a bridge)!

Gently

Pull

Gently

Pull

Turn your structure upside down.  Does the wire stiffen it?

Yes.  The wire is under tension this time, and it pulls back to stiffen the structure.

Replace the wire with a half strip.  Test it upside down and right side up.

Trusses: structures that make bridges stiff and stable.

Deck: the surface cars, trains, people, and pipelines use to cross a bridge.

You’re almost ready to design your own bridge!  Follow these last few steps to turn your truss, from Step 17, into a bridge.

Your second truss will need nuts to rest on; add a nut to each screw.

You may need to adjust your nuts so that your trusses stay parallel.

Create the second truss by placing half strips on the nuts.

You can use a screwdriver 
and pliers to make your 
joints tighter.

Tip

Finish the bridge by tightening nuts onto the screws.

DISTANCE CHALLENGE

Use chairs or desks as abutments on each end of the bridge.  Piers (middle supports) are not allowed.

Nothing may be used to hold the bridge on the abutments (e.g., no tape, weights, etc.).

Span

(distance between abutments)

Your bridge must support a shoe (or teacher-approved weight) above the ground.

CRITERIA:
(what your design must do)

Redesign your bridge to make it as long as possible.  The bridge with the longest span wins!

Test it, change it, and repeat!

CONSTRAINTS:
(rules and limits for your design)

Components:  You may only use the components listed on Page 1 – you can’t add extra parts.  Connector strips can be cut or left whole.

Use the optional Design Grid and Engineering Notebook to plan and document your designs!  Documents available at teachergeek.com/bridges

STRENGTH CHALLENGE

(optional)

52.5 cm
(20.5 in)

CRITERIA:

Redesign your bridge to hold the most weight possible.

The bridge that holds the most weight wins!

Hang a bucket near the middle of your bridge, then fill with water bottles or other weights.

CONSTRAINTS:

Your bridge must hold the weight over a span of 52.5 cm (20.5 in).

All other constraints 
are the same as the Distance Challenge (above).

Want more guidance for Strength Testing? Get the Testing Guide at teachergeek.com/bridges

These types of bridges can be built with TeacherGeek components, but they cannot be tested on a normal testing station.

What type of bridge will you build?

Cantilever Bridges – Use cantilevers – structures that are supported on one side, like a diving board.  A cantilevered bridge would be built like two diving boards, which are connected in the middle after each side is built.

Suspension Bridges – Use cables to support the deck.  Main cables are connected to towers and anchored to the ground.  Suspension cables support the deck from the main cables.

Truss Bridges – Use trusses to carry a load.  Trusses are typically made of triangles.  Other bridge types may incorporate trusses, too.

Arch Bridges – Use arches, or circular shapes, to carry a load.  Arches are very strong in compression, and can go over, under, or through the bridge deck.

Cable Stayed Bridges – Use cables (called 
“stay cables”) attached to towers to carry a load.