Have you made an electromagnet? You will need to make it, before making your crane. Instructions for making the electromagnet are at teachergeek.com/learn

Other documents available at teachergeek.com/learn

Here’s how you make stuff with TeacherGeek:

Adult supervision required for children 12 and under.

You will need these TeacherGeek components for the Crane:

Available in the TeacherGeek Electromagnet Crane Activity, TeacherGeek Maker Cart, or at teachergeek.com
(activity packs include extra components for further tinkering and innovation).

| 1- Connector Strip | 1- Dowel 300mm (12in) | 1 - Hole Plate | 1 - Block |
| --- | --- | --- | --- |
| 2 - 13mm Screw #6 (¼in) | 2 - 2.5cm Screw #10 (1in) | 1 - Nut #10 | ≥ 2cm Slide Stop (≥ ½in) |

You will need these tools, they can be shared:
Tools available at teachergeek.com

| Multi-Cutter SKU 1823-81 | Screwdriver SKU 1823-90 | Pliers SKU 1823-86 | Reamer SKU 1823-87 |
| --- | --- | --- | --- |

You will need these non-TeacherGeek supplies:

| Tape | Magnetic Materials Erasers, Washers, Stapes, Dimes Candy, Pennies, etc. |
| --- | --- |

|  | Cut a 12cm dowel. 12cm |  | Tap or push the dowel into a corner of a hole plate. |
| --- | --- | --- | --- |
|  | Ream one hole of a block. |  | Cut two 1cm sections of slide stop. 1cm 1cm |
|  | Place one of the slide stop sections onto the dowel. |  | Put the dowel through the reamed block hole. Use a slide stop section to hold it on. |
|  | Use a 2.5cm (1in) screw to attach a connector strip to the block. This will become the crane arm. |  | Use a 2.5cm (1in) screw and nut to attach your electromagnet to the arm. |
|  | Finish your example Electromagnet Crane by attaching the battery holder and taping loose wires. |  |  |

Use tape to hold loose wires

Or use zip ties

Documents at teachergeek.com/learn

This example crane works okay, but you can make it work much better. 
It is time for you to redesign it. 
Start onto an Electromagnet Crane Engineering Challenge.

Screw the battery holder to the hole plate, from the bottom.