Hydraulics are AMAZING!
Use the power of water to control a machine.

This guide will help you create 
your own Advanced Hydraulic Arm.

Perfect for Grades: 7+
Difficulty: Advanced

Download Documents
at teachergeek.com

THINKING AHEAD
What kinds of objects can you
move with different gripper designs?

Below is the list of “ingredients” you’ll need for each Hydraulic Arm Build.
Available as single: SKU 1823-08 or 10 pack: SKU 1823-09. Both include extra parts for your own innovative creations!

| x5 | x5 | x3 | x4 | x7 |
| --- | --- | --- | --- | --- |
| Connector Strips | 300mm (12″) Dowels | Hole Plate | Tubing: 90cm (36”) - x1 60cm (24”) - x1 40cm (15”) - x2 | Perpendicular Blocks |
|  |  |  |  | x1 x1 x12 x24 x14 |
| #10 1.5″ Screw | #10 1″ Screw | #6 ½″ Screw | #10 Nut | #10 Locking Nut |
| x1 | x1 | x7 | x1 | x10 |
| 100mm (3″) Slide Stop | Cable Tie | 14mL Cylinder | 4.5mL Cylinder | Small Rubber Bands |

Perfect for sharing in groups of 3 and 4!

Time to break out those tools and start building! Remember to be kind and share with others.

| Multi-Cutter | Reamer | Screwdriver | Pliers |
| --- | --- | --- | --- |

Go on your own scavenger hunt to find these items. Try building with all kinds of materials!

| Safety Goggles | Crayon (rub on dowels to make sliding them easier into holes of components) | Tape | Recycling Materials What else could you use for a gripper? |
| --- | --- | --- | --- |

14ml

14ml

14ml

4.5ml

14ml

14ml

14ml

14ml

40cm (15”) Tubing

40cm (15”) Tubing

60cm (24”) Tubing

90cm (36”) Tubing

Hydraulic systems are the foundation of how this build design functions. Follow the steps below to connect and fill cylinders to create your own hydraulic system.

Please note: in order to work properly, there can be no air bubbles in the cylinders or tubes.

|  | Cut tubing into the following lengths: If you’re using an activity pack, the tubing has been cut for you. 40cm (15″) 60cm (24″) 90cm (36″) |  | Fill the seven 14mL cylinders and one 4.5mL cylinder with water by pushing the piston all the way in and submerging the tip under water; pull back to fill completely with water. Remember: no air bubbles Quick Tip Use food coloring to make seeing the water easier. |
| --- | --- | --- | --- |
|  | Attach the cut tubing pieces from Step 1 to four of the filled cylinders from Step 2. Connection Close-Up |  | Over a cup, push the cylinder piston to completely fill the tubing with water. Remember: no air bubbles |
|  | Attach the water filled tubing to the remaining water filled cylinders from Step 2. |  | Insert a #6 ½″ screw into the hole aside the cylinder’s tip to prevent the tubing from disconnecting. |
|  | From the end of a dowel, measure and cut three 50mm (2″) sections - save the extra, you will use this later. |  | Insert the cut dowels from Step 7 into the outside corners of a hole plate as shown below. |
|  | Slide a block halfway onto the corner dowel piece. |  | Push another hole plate onto the dowels from Step 3, sandwiching them in place. This piece will be the waist for your arm. |
|  | Cut two 60mm (2 ⅜″) dowels and insert into two connector strips as shown below to create the main boom. |  | Cut a 65mm (2 ⅝″) dowel and slide into the hole plates as shown. |
|  | Secure the main boom to the arm waist by tapping or pushing the connector strips around the dowel from Step 12. This creates the boom fulcrum or pivot point. | 40cm Tubing | With one of the 40cm (15″) hydraulic systems from Steps 1-6, insert one of the cylinder ends inside the two hole plates. 40cm Hydraulic System |
|  | Ream the hole shown on the cylinder’s piston, cut a 60mm (2 ⅜″) dowel and slide through the hole. |  | Spread the main boom apart slightly and insert the dowel from Step 15 into the connector strips. |
|  | This is the shoulder for your arm. The boom should move as the cylinders are pushed and pulled. |  |  |
|  | Cut a connector strip in half 15 cm (16”) to get two equal pieces. Since connector strips do not have a center hole, cut as shown below. |  | Use four #10 1″ screws and #10 nuts to attach the connector strips from Step 18 to a hole plate. |
|  | Cut a 170mm (6 ⅝″) dowel and slide into the last holes of the connector strips. This forms the base of your arm. |  | Feed a #10 1″ screw up from the bottom and through one of the back-center holes of the base’s hole plate. |
|  | Attach the arm’s waist to the base by feeding the screw from Step 21 through the center hole of the block from Step 9 and fasten the #10 locking nut to the screw to hold the waist to the base. The screw and nut should be loose enough to allow the waist to easily turn on the base. |  |  |
|  | Create the forearm by cutting two 40mm (1 ⅝″) dowels and inserting them in between two connector strips. |  | Ream the two holes marked with a . |
|  | Cut a 60mm (2 ⅜″) dowel and slide it into the reamed holes. Slide |  | Cut slide stop into two 5mm (⅜″) sections and slide it onto the outside of the 60mm dowel. This becomes the forearm. |
|  | Insert the forearm dowel inside the connector strips of the main arm to create the elbow. 60cm Tubing |  | Use the of the 60cm (24″) hydraulic systems from Steps 1-6, and ream the hole on the piston marked with a . |
|  | Cut a 60mm (2 ⅜″) dowel and slide it into the reamed hole of the piston from Step 28 and secure with two sections of slide stops. |  | Insert the dowel end of the piston to the top of the main arm and the pegs of the cylinder to the top of the forearm. |
|  | Cut a 75mm (3″) dowel and push, wiggle, or tap into the corner of the base’s hole. Top with a block. | 40cm Tubing | Use the remaining 40cm (15″) hydraulic systems from Steps 1-6, and ream the hole on the piston marked with a . |
|  | Insert the pegs of the cylinder in between the base’s hole plate and the perpendicular block from Step 31. |  | Place a #10 1.5″ screw through the hole plate of the waist and secure with a #10 nut. |
|  | Cut a 60mm (2 ⅜″) dowel and tap into the outer hole of a perpendicular block. |  | Insert the dowel from Step 35 into the reamed hole of the cylinder’s piston and attach the block (through the center hole) to the screw by using two #10 nuts on each side of the block. |
|  | Cut the pegs off the three cylinders that are attached to your arm and use #10 1″ screws and #10 nuts to attach the cylinders together to form a control panel. | 14ml 90cm (36”) Tubing | Let’s create an end effector: a device or tool that is connected to the end of a robot arm (as a hand) and remains controlled by a hydraulic system. Use the remaining hydraulic system from Steps 1-6 (the 14mL cylinder connected to the 4.5mL cylinder) and cut off the pegs of the 14mL cylinder and screw it to the control panel as well. Design & Create Your Own End Effector Think of what tasks your robot could perform, such as gripping, suction, scooping, dispensing, or welding. Download at teachergeek.com/learn Now is the time to Design & Create your own end effector. Use guide as example. |
|  | End Effector Options: Create this example gripper (end effector). Create your own end effector. Create the example gripper and then change it into your own end effector. Remember: your end effector should be designed to complete a specific task. |  | Start to create the gripper by cutting two 90mm (3½″) dowels and push, wiggle, or tap into the center holes of two blocks. |
|  | Cut two 110mm (4⅜″) dowels and push, wiggle, or tap into the outside holes of one of the blocks from Step 38. |  | Ream the holes marked with the symbol on the remaining open block from Step 38. |
|  | Slide the reamed block from Step 40 onto the 110mm (4⅜″) dowels from Step 39. |  | Insert the two dowels into the holes of the 4.5mL cylinder. |
|  | Use a cable tie to attach the cylinder’s piston to the dowel with the sliding block. |  | Place two blocks on the ends of the 60mm (2 ⅜″) dowels to complete the gripper. |
|  | Cut the last two holes off the connector strips of the forearm. |  | Use a screw and nut to attach the gripper to the forearm. |
|  | Congratulations! Now try your design in the Lab or Design & Engineering Challenge. Quick Tip Use different markers, tape, or colored water in the cylinder tubing to identify which control panel cylinders connect to which arm parts. |  |  |

Create your own end effector to complete
all kinds of tasks. Try building an end effector
that will perform gripping, suction, scooping, 
dispensing, or welding actions.

Use TeacherGeek components and 
recycling materials to achieve your goal.