Hydraulics are AMAZING!
Use the power of water to control a machine.

This guide will help you create 
your own Hydraulic Arm.

Swing!

GRAB!

Swing!

Download Documents
at teachergeek.com

THINKING AHEAD
What kinds of objects can you move with different gripper designs?

Below is the list of “ingredients” you’ll need for each Hydraulic Arm Build. 
Available as a 10 pack, which includes extra parts for your own innovative creations!

| x6 | x5 | x6 | x1 | x1 | x1 |
| --- | --- | --- | --- | --- | --- |
| Connector Strips | 300mm (12″) Dowels | Blocks | Cable Tie | Tubing: 40cm (15”) – x2 90 cm (36”) – x1 | 100mm (3″) Slide Stop |
| x5 |  | x6 |  |  | X1 x18 x5 |
| #10 1″ Screw | ¼” Cylinder Screw | #10 Nut | 14mL Cylinder | 4.5mL Cylinder |  |

Perfect for sharing in groups of 3 and 4!

Time to break out those tools and start building! Remember to be kind and share with others.

| Multi-Cutter | Reamer | Screwdriver | Pliers |
| --- | --- | --- | --- |

Go on your own scavenger hunt to find these items. Try building with all kinds of materials!

| Safety Goggles | Tape | Crayon (rub on dowels to make sliding them easier into holes of components) | Recycling Materials What else could you use for a gripper? |
| --- | --- | --- | --- |

40cm (15”) Tubing

40cm (15”) Tubing

90cm (36”) Tubing

14mL

14mL

4.5mL

14mL

14mL

14mL

Hydraulic Systems are the foundation of how this build design functions. Follow the steps below to connect and fill cylinders to create your own hydraulic system.

Please note: in order to work properly, there can be no air bubbles in the 
cylinders or tubes.

|  | Cut tubing into the following lengths: If you’re using an activity pack, the tubing has been cut for you. Two 40cm (15″) One 90cm (36″) |  | Fill the five 14mL cylinders and one 4.5mL cylinder with water by pushing the piston all the way in and submerging the tip under water; pull back to fill completely with water. Remember: no air bubbles Quick Tip Use food coloring to make seeing the water easier. |
| --- | --- | --- | --- |
|  | Attach the cut tubing pieces from Step 1 to four of the filled cylinders from Step 2. Connection Close-Up |  | Over a cup, push the cylinder piston to completely fill the tubing with water. Remember: no air bubbles |
|  | Attach the water filled tubing to the remaining water filled cylinders from Step 2. |  | Insert a 1/4″ cylinder screw into the hole next to the cylinder’s tip to prevent the tubing from disconnecting. |
|  | From the end of a dowel, measure and cut three 55mm (2 ⅛″) sections - save the extra, you will use this later. |  | Push, wiggle or tap the 55mm (2 ⅛″) dowels into the holes of a connector strip as shown below. |
|  | Push, wiggle or tap a second connector strip onto the 55mm (2 ⅛″) dowels. |  | Ream the two holes marked with a . |
|  | Cut a new dowel to get two 150mm (6″) dowels and slide one through the reamed holes from Step 10. Hold in place with two cut 6mm (¼″) sections of slide stop. |  | Push, wiggle or tap two blocks onto each end of the 150mm (6″) dowel and leaving 5mm (¼″) of space on the ends. |
|  | With the blocks pointing down, attach the ends of two connector strips to each side of the 150mm (6″) dowel. Full (uncut) Strip | Full (uncut) Strip | Push, wiggle or tap the leftover 150mm (6″) dowel from Step 11 in between the ends of the connector strips from Step 13 to complete the base. |
| Good job! You just completed the base and first boom of your hydraulic arm. You should be able to pivot the boom at the fulcrum. |  |  |  |
| 14mL 14mL 40cm (15”) Tubing | With the 40cm (15″) hydraulic systems from Steps 1-6, ream the hole marked with a on one of the cylinder’s pistons. |  | Cut a 150mm (6″) dowel and insert through the reamed hole from Step 15. |
|  | Cut two 6mm (¼″) sections of slide stop Slide onto both ends of the 150mm (6”) dowel all the way to the center, touching the piston’s handle. |  | Insert the 150mm (6″) dowel in between the base connector strips and insert the pegs of the cylinder in between the boom connector strips. (You can change this position later). |
| Quick Experiment! Does changing the placement of the piston move the lever with the same distance or force? |  |  |  |
|  | Create a second boom by cutting two 45mm (1¾″) dowels and inserting them between two connector strips. |  | Ream the four holes marked with a . |
|  | Cut a 65mm (2½″) dowel and slide through the reamed holes to connect the first and second boom. | 14mL 14mL 90cm (36”) Tubing | With the 90cm (36″) hydraulic systems from Steps 1-6, ream the hole marked with a on one of the cylinder’s piston. |
|  | Cut a 45mm (1¾″) dowel and insert into the reamed hole from Step 22. Hold in place with two cut 6mm (¼″) sections of slide stop. |  | Place the cylinder assembly from Step 23 in between the first and second boom connector strips (you can change this position later). |
| Quick Experiment! Change where the cylinders attach to your arm to allow it to move as far in and out as possible. |  |  |  |
|  | Start to create the gripper by cutting two 90mm (3½″) dowels and push, wiggle, or tap into the center holes of two blocks. |  | Cut two 110mm (4⅜″) dowels and push, wiggle, or tap into the outside holes of one of the blocks from Step 25. |
|  | Ream the holes marked with the symbol on the leftover block/dowel from Step 25. |  | Slide the reamed block from Step 27 onto the 110mm (4⅜″) dowels from Step 26. Slide |
|  | Use the remaining hydraulic system from Steps 1-6 (the 13mL cylinder connected to the 4.5mL cylinder) and insert the two dowels into the holes of the 4.5mL cylinder. 4.5mL Cylinder |  | Use a cable tie to attach the cylinder’s piston to the dowel with the sliding block. |
|  | Place two blocks on the ends of the 60mm (2 ⅜″) dowels to complete the gripper. |  | Cut the last two holes off one of the connector strips of the second boom. |
|  | Use a screw and nut to attach the gripper to the second boom. |  | Cut the pegs off the loose cylinders and use #10 1″ screws and #10 nuts to attach the cylinders together to form a control panel. |
|  | Tape the tubing to the arm, but be careful not to crimp or smash the tubing. |  |  |

Quick Tip
Use different colored markers,
tape, or colored water in the cylinder tubing to identify which control panel cylinders 
connect to which arm parts.

Documents at teachergeek.com/learn

If you are going 
to do the optional 
Fluid Power Lab, 
now’s the time!