Start by building the example launcher, then turn it into your own unique design.

Download documents at teachergeek.com/learn

For use with TeacherGeek Projectile Launcher Activity, 
or Maker Cart available at teachergeek.com.

TeacherGeek components for the example projectile launcher, 
and extra pieces to turn it into your own unique design.

| 10 - Blocks | 5 - Dowels 30cm (12″) | 2 - Connector Strips | 1 - Hole Plate |  | 1 - Ping Pong Ball |
| --- | --- | --- | --- | --- | --- |
| 2 - Nuts #10 | 2 - Lock Nuts #10 | 1 - 38mm Screw 38mm (1½″) #10 | 1 - 25mm Screw (1″) #10 |  | 10 - Rubber Bands |
| 2 - Stop Clips | 1 - Slide Stop 76mm (3″) | 1 - Metal Wire 7.5cm (3″) | 1 - Plastic Coated Wire 8cm (3¼″) | Cut from Wire Roll |  |

►

Components available in the TeacherGeek Ping Pong Launcher Activity, 
TeacherGeek Maker Cart, or at teachergeek.com

This isn’t a kit. You’re going to really build (cut, ream, screw) your launcher. 
Here are the tools you’ll need.

| Multi-Cutter SKU 1823-81 | Reamer SKU 1823-87 | Screwdriver SKU 1823-90 | Pliers SKU 1823-86 |
| --- | --- | --- | --- |

Tools available at teachergeek.com

You will need these non-TeacherGeek supplies:

| Tape Masking, Painter’s, Duct; Any kind of tape will work. | Safety Goggles Should be worn during the activity. Inventing can be projectile! | Recycling Materials Add to your design | Scissors For cutting out the protractor and recycling materials. |
| --- | --- | --- | --- |

TeacherGeek Protractor

Print on cardstock or thicker paper for a sturdier protractor.

Printable Protractor Download:

http://teachergeek.org/protractor-angle_finder.pdf

| 13cm | 5cm Cut a 13cm (5”) and 5cm (2”) dowel. |  | Push or tap each dowel from Step 1 into a block as shown, so the dowels stick out a little. 5cm Block 13cm |
| --- | --- | --- | --- |
|  | Push or tap the dowels from Step 2 into the hole plate, as shown. Hole Plate Tap! |  | Rub a crayon or bar of soap on two full (30cm) dowels. This helps the dowels slide. 30cm dowel Slide the dowels Launch Pad from Step 4 through the block holes hanging off the hole plate. |
| TeacherGeek Protractor | Print, cut, and tape your protractor to the launcher, as shown. 7.5cm Metal Wire | Use a dowel or pencil to bend the hook. 6.5cm | Poke! Make sure the metal wire points straight down when the protractor is tilted. Bend a 1.5cm hook on the 7.5cm metal wire. Poke it through the protractor at the . Hang it on the paper as shown. |

Launch Pad

Don’t cover the 
cross-hair (circle) 
with tape.

Change the angle of your launch pad 
by sliding these dowels up and down.

Check the angle using the protractor.

►

Printable Protractor Available at:

http://teachergeek.org/protractor-angle_finder.pdf

Congratulations! 
Your Launch Pad is finished!
Now, it’s time to create 
the Launching Mechanism.

|  | Cut a piece of connector strip, four holes long. 4 holes |  | Loop a rubber band through each hole on either side. Rubber Band Loop! Getting it right can be tricky. Take your time! |
| --- | --- | --- | --- |
|  | 8cm Wire Piece Cut, or find a 8cm (3”) piece of plastic coated wire. Bend it in half and put the ends through the connector strip, as shown. |  | Twist wire ends together to create a loop. Wire |
|  | Slide the rubber bands over the inner dowels of your Launch Pad from Step 5, as shown. Rubber Bands Congratulations! Your Launch Mechanism is finished. Now, let’s create a trigger to fire a ping-pong ball. |  |  |
|  | On the other side of the hole plate, put a 38mm screw through the underside (bottom) of the plate. Fasten with a nut. Tighten! Nut 38mm screw Top View Underside of Plate |  |  |
|  | Push a stop clip on the screw to create your trigger. Trigger Stop Clip |  | Lock Nut Cap the screw with a lock nut. This will prevent the stop clip from sliding off. |

- Pull back the launching mechansism 
and attach it to the trigger (stop clip).

- Place a ping-pong ball 
into the launch mechanism.

- Turn the trigger (stop clip) to fire!

Tip: Make sure trigger is lined-up with the center 
of the ball.

Pull!

Caution: Wear safety glasses. Never launch at another person.

►

You Did It! 
Your example Launcher is finished, 
but its design works just okay.
You can make it so much better.

Your launcher should launch balls with precision and accuracy.

Precision is how consistent (repeatable) your results are.

Accuracy is how close you get to the actual (true) target or goal, such as the center of a target.

The example launcher is a little wimpy. Can you make your launch 
mechanism and trigger more powerful, as well as accurate and precise?