For use with TeacherGeek Wind Lift Activity Pack, or Maker Cart. Find documents and activity materials at teachergeek.com.

Name(s): _______________________________________________________________________

Make sure you have built a TeacherGeek Wind Lift, before starting this lab.

Energy is the ability to do work

Potential energy is stored energy. It is as the result of its position.

Kinetic Energy is the energy of motion. If an object is moving, it has kinetic energy.

The fan is off. The bucket is lowering.

The fan is blowing. 
The bucket has lifted.

Write the letters to match the type of energy shown above. Use letters only once.

1.  ____ shows potential energy turning into kinetic energy

2.  ____ shows potential energy as a result of height

3.  ____ shows kinetic energy that came from electricity

4.  ____ shows the kinetic energy of blowing air; from the potential energy of the bucket

5. Put something heavy in the bucket. Wind up the bucket by hand (without the fan) and let it drop. Notice how fast the blades spin and 
how much air is pushed. Now put something light in the bucket. 
Wind it up and let the bucket drop. What’s different? Figure it out. 
Explain it to your teacher using these, and other words:

□ Potential Energy □ Kinetic Energy □ More Energy □ Less Energy

Teacher Signature _______________________________________

Name(s): _________________________________________________________

Make sure you have built a TeacherGeek Wind Lift, before starting this lab.

1. Hypothesis: How do you think changing the angle of the 
Wind Lift Blades will affect the number of pennies it can lift?

Many wind turbines change the angle of their blades to adjust to wind conditions.

__________________________________________________________________

__________________________________________________________________

__________________________________________________________________

| Test your Hypothesis Test how many pennies your wind turbine can pick up, at different blade angles. Use a protractor to set blade angles. |  | Change the Blade Angle A. Loosen the hub screw a little bit, so the blades can turn, but do not fall. B. Change the blade angle using a protractor. C. Tighten the screw. |  |
| --- | --- | --- | --- |
| 0° Blade Angle | 30° Blade Angle | 60° Blade Angle | 90° Blade Angle |
| 2. What happens when the blade is at 0°? _____________________ _____________________ _____________________ _____________________ | 3. How many pennies can it lift at 30°? _____________________ 4. How long does it take to lift the bucket? _____________________ | 5. How many pennies can it lift at 60°? _____________________ 6. How long does it take to lift the bucket? _____________________ | 7. What happens when the blade is at 90°? _____________________ _____________________ _____________________ _____________________ |

8. Was your hypothesis correct? Please explain why, or why not (don’t just write “yes” or “no”).

__________________________________________________________________________________________________

__________________________________________________________________________________________________

__________________________________________________________________________________________________

9. The wind turbine shown on the right was damaged from high winds. What did you learn from your blade angle tests that could have helped this wind turbine? What would you have it do in high winds to protect the blades?

________________________________________________________________________

________________________________________________________________________

________________________________________________________________________

________________________________________________________________________

10. Write the names of the components shown below. 	Note: Blade is not an answer.

- ____________________________________________

- ____________________________________________

____________________________________________

These protractors work well 
for the Wind Lift Angle Lab. Print them on heavy paper and cut them out. The notch allows them to position centrally on the blade shaft.

Documents at teachergeek.com/learn

These protractors can be 
used with the optional 
Design & Engineering Challenges as well.

Some wind turbines/wind mills have blades with a lot of area. Other wind turbines have blades with very little area.

Name(s): ________________________________________________________________

Make sure you have built a TeacherGeek Wind Lift, before starting this lab.

1. Hypothesis: How do you think the Wind Lift blade area affects the number of pennies that can be lifted?

__________________________________________________________________

__________________________________________________________________

__________________________________________________________________

__________________________________________________________________

Get Ready

Make sure that your blades are 23cm x 5cm. If they are not, cut new blades and tape them on. They should be like this.

23cm

5cm

Blade Area:

23 x 5 = 115cm2

115cm2 x 3 = 345 cm2

Blade Area x # of blades = total area

Set the angle of your blades to approximately 30°.

Change the blade angle: 
A. Loosen the hub screw a little bit, so the blades can turn, but do not fall out. B. Change the blade angle using a protractor. C. Tighten the screw again.

Test your Hypothesis: How does blade area affect the number of pennies that can be lifted?

| Use your 5cm wide blades. | Cut your blades to 3cm wide. | Cut your blades to 1 cm wide. |
| --- | --- | --- |
| 5cm | 3cm | 1cm |
| 2. What is the combined area of all of the blades? _____________________________ 3. What is the maximum number of pennies that can be lifted? _____________________________ 4. How long does it take to lift the bucket? ____________________ seconds | 5. What is the combined area of all of the blades? _____________________________ 6. What is the maximum number of pennies that can be lifted? _____________________________ 7. How long does it take to lift the bucket? ____________________ seconds | 8. What is the combined area of all of the blades? _____________________________ 9. What is the maximum number of pennies that can be lifted? _____________________________ 10. How long does it take to lift the bucket? ____________________ seconds |

11. Was your hypothesis correct? Please explain why, or why not (don’t just write “yes” or “no”).

__________________________________________________________________________________________________

__________________________________________________________________________________________________

__________________________________________________________________________________________________

__________________________________________________________________________________________________