Download classroom documents at teachergeek.com/learn

For use with TeacherGeek Maker Cart 
available at teachergeek.com

Components available in the TeacherGeek Maker Cart, or at teachergeek.com

|  | 1 - Hole Plate | 1 – Cylinder 4.5 mL |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 4 - Barrels 14.5ml | Vinyl Tubing ⅛″D x 38cm (15″)L |  |  |  | Portion Cup |  |
| 6 - Dowels 300mm (12″) | 4 - Connector Strips | 1 - Slide Stop 7.6 cm (3”) | 4 - Blocks |  | 2 - Check Valves | 1 - T-Connector |

Tip: Save all your materials (even what you cut off). Keep them in a bag. They can be used later.

Tools can be shared between classes and groups.

| Multi-Cutter | Hammer | Reamer |
| --- | --- | --- |

Here are some non-TeacherGeek materials that you will need.

| Recycling Materials Plastic, cardboard, food packaging. | Growing Media Piece of cloth, rockwool, cotton ball, etc. Needs to hold seedling in barrel, but still allow roots to grow. | Seeds mung beans, tomatoes, lettuce, spinach, basil |
| --- | --- | --- |

Use hydroponic systems to grow plants, soil free.
With so many choices, which will you build?

or

or

Drip water on plants,
collect for reuse.
How many drip systems could you build at once?

Flood your plants,
then drain for reuse. How long will your 
plant stay submerged?

Flood & Drain

Compound

Drip

Mix and match to
combine designs.
What system will you design for your plant?

This is not a step-by-step guide -
start with an example build, then 
design your own hydroponic system. 
The hole plate can fit two systems, 
so mix and match, add an optional check-valve, water aerator pump or 
enclose your system in a greenhouse.
There are no mistakes; just ways
to make your design even better!

              Now go, pick your system 
             and get building!

Ideal hydroponic plants grow quickly, and don’t require too much surface area to thrive.

- mung beans

- lettuce

- tomatoes

- basil

clean sock

rockwool

cotton ball

Hydroponics require a neutral growing medium 
for support, while allowing oxygen and water in.

Try rockwool, cotton balls, perlite, 
       peat plug or even an old clean sock.

Place your seed in the growing media and moisten with water. Store in a 
dark, warm place and re-moisten daily.

seed

seedling

Allow your seed to germinate (sprout) over the next few days into a seedling (green shoots and exposed leaves).
Now it’s ready to be ‘planted’.

Nutrient Solution Prep

Feed me Seymour! Plants require both macro and micro-nutrients,
which you must prepare as a nutrient solution (water + nutrients).

Commercial Solutions (e.g. FloraMicro, FloraGro) contain 
Nitrogen, Potassium, Phosphorus and trace minerals like calcium.
DIY: Experiment with old tea bags or recycled fish tank water.

Flood & Drain

Insert two dowels into a hole plate and push on two barrels as shown, 
one higher, one lower.

Cut a 3cm (1.1”) dowel and 
tap or push it into a block.

Push the block onto an upright dowel and attach the barrel to 
the shorter dowel so it won’t slide.

Ream the barrel holes 
very well, to allow 
easy adjustment 
on the dowels.

3cm

Cut a 3cm (1.1”) piece
of slide stop. Slide it 
10 cm (3.9”) from the bottom of a dowel.

upright dowel

3cm slide stop

cut

Place your seedling 
in the barrel without 
the slide stop.

Now you’re ready to Flood & Drain your Hydroponic System.

Attach 16cm (6”)
of tubing to connect 
your barrels. Make 
sure you can adjust
your barrel up and
down the dowel.

16cm

Congratulations! You’ve built an example Flood & Drain System.
It only works ok. You can make it so much better.

Flood & Drain Options

Flood & Drain Systems
Rockwool and gravel 
growing media work 
best. Be careful not to 
over-flood your plant – 
this could lead to algae, 
or suffocate the plant.
How could you add 
oxygen to the solution?

A

Drain

Attach the drip barrel below the plant, right on the slide stop, to drain
nutrient solution.

Flood

Attach the drip barrel above the plant to flood with
nutrient solution.

B

0

C

Fill
Use a cylinder or eye dropper to fill the barrels with fluid.

Drip

Insert two dowels into a hole plate and push on two barrels as shown, 
one higher, one lower.

Slide your seedling into the top of 
the lower barrel. If needed, adjust 
your growing media so the roots are 
exposed below and the sprout above.

above

below

Fill your top barrel with water to give 
your plant some much needed hydration.
You can add tubing to your lower barrel 
for a recovery drip system into the cup.

Cut an 8cm (3”) section of tubing and tie a knot. 
Insert into a barrel. 
How loose or tight 
you tie the knot will affect the drip speed.

Place a portion cup below to collect 
water runoff.

Ream the barrel holes 
very well, to allow 
easy adjustment 
on the dowels.

Congratulations! You’ve built an example Drip System.
It only works ok. You can make it so much better.

Drip Options

Top Option

Insert the drip tube above the plant to slowly give water.

Fill
Use a cylinder or eye dropper to fill the barrels with fluid.

Quick Tip: 
Check the knot often 
to assure fluid is flowing 
through. Try a clamp 
as another method 
to control the drip.

Bottom Option

Insert the drip tube below the plant to slowly remove water.

Recovery Drip System:
Collect excess water for re-use. 
What other water conservation 
designs can you create?

Documents at teachergeek.com/learn

If you are going  
to do the optional 
Challenges and Labs, go there now.

Engineering Notebook:
Use engineering notebook pages to go through the Design Process. Sketch ideas, take notes and use them to compete in challenges!

Documents at teachergeek.com

Compound

Piston

Hydroponic Systems need 
fluids to thrive – nutrient solution 
(water + nutrients) and oxygen. 
Aeration is a method of pumping 
air (oxygen) into water and 
removing unwanted gases.

Otherwise, the nutrient solution 
can easily become stagnant 
(no flow), which could lead to 
algae blooms or suffocation.
Can you design a way to aerate 
and control the flow of fluid?

Use an empty cylinder 
as an oxygen pump.

Push the piston up and down to move the fluid.

Check Valve

Check Valve

T-Connector

Fluid 
Reservoir 
(Unaerated)

Aerated 
Fluid

T-Connectors: allow fluid to flow between three ports (openings).

Check Valves: allow fluid flow
(liquid or gas) in one direction.

Use These TeacherGeek Materials

+

4.5 mL Cylinder

Your Hydroponic
System Design

Two 600mm (2 ft)
tubing sections

T-Connector

Check Valve

Greenhouse: an enclosed glass or transparent building for gardening, allowing in sunlight, while also trapping heat inside.

Light Energy

Recycling With Purpose
Greenhouses are built out
of all sorts of things. Plastic soda bottles make great materials to trap heat.

Greenhouse Effect: The earth’s atmosphere acts like a giant greenhouse. Greenhouse Gases (water vapor, carbon dioxide, methane, ozone) 
absorb and trap heat, preventing the planet from freezing.

Earth needs a careful balance – 
            too many greenhouse gases could 
 make the climate too hot (due to pollution, 
deforestation and the burning of fossil fuels).

DIY Greenhouse
Greenhouses can be built
from more than just glass.
Design a (micro)greenhouse.
Try all sorts of transparent,
recycled materials to trap 
heat and absorb light.

Cut and paste this inspiration, 
along with sketches and notes,
in an Engineering Notebook 
page to improve your design.