Build an example claw, then experiment and evolve your own unique design!

TRUE

STEM

STEAM

Check out our Hydraulic Claw Videos by scanning the QR Code or going to teachergeek.com/claw

You Are Here

Start here! Build your Claw, evolve your design, and begin the Ocean Cleanup Challenge!

Optional Lab

Optional Challenges

-Fluid Power Lab 
 (Ages 12+)

-Claw Ball Challenge*
-HORSE Challenge*

*See Page 10

Go Guide

Choose how you would like to complete this activity.
Download documents & videos at teachergeek.com/claw

Supplies

Modify materials to make even more creative designs with the Maker Tool Set

SKU 1823-84

Optional Tools

String

Screwdriver

A Container
To hold water for filling cylinders

Tape

Cylinder Screws
SKU 1821-21

2

Strips

30 cm (12 in)

SKU 1821-31

4

Dowels

various sizes

SKU 1821-20

12

Slide Stop
7 cm (3 in)
SKU 1821-49

1

Screws
25 mm (1 in)
SKU 1821-22

8

Nuts
#10 hex
SKU 1821-25

4

Blocks
SKU 1821-34

4

Cylinders
4.5 ml
SKU 1821-52

2

Tubing
38 cm (15 in)
SKU 1821-51

1

Zip Ties
1823-50

4

PICTURE

NAME

QTY

Dowel Sizes

2x 15 cm (6”)
4x 7.5 cm (3”)
6x 5 cm (2”)

Claw Parts

These are the parts you need to build one claw, plus some extra parts for your own unique designs.

Recycling Materials
What can you use for 
your claw grippers?

Materials You Supply

Want to learn more about hydraulics?

Download the 
Fluid Power Lab at teachergeek.com/claw
Ages 12+

# 5

# 4

# 3

# 2

# 1

Food coloring makes water easier to see.

Optional Tip

Your hydraulics are done!  Test them out.

Insert a cylinder screw into each cylinder to secure the tubing.

Remove all bubbles from cylinders & tubing for best performance.

Tip the cylinder so bubbles rise 
to the base of the tube.

Push the air out and refill.

Attach the tubing to the other cylinder.

Fill the tubing completely with water by pushing the piston all the way in.

Push & Pull

Attach 38 cm (15 in) of tubing to just one filled cylinder.

Fill both cylinders with water.

Place tips under water.

Pull pistons to completely fill with water.

Fill The Hydraulics

Tubing longer than 38 cm (15 in) is harder to fill.

After you make your claw, try a Challenge! (Page 9-10)

Accordion
(Page 4)

These examples get you started, then you can experiment and evolve your own unique design!

Which Claw Will You Make?

Pincer
(Page 8)

Chopsticks
(Page 6)

Accordion Example

Cross two strips and add a screw near the middle.

# 2

# 4

# 3

# 1

Full Strip

Make two half strips, if you don’t already have them, by cutting or snapping a full strip.

Half Strips

2x

This mechanism moves like an accordion.

Test your mechanism!  
Next, you are going to add hydraulics.

Add the half-strips and screws to 
make an accordion-style mechanism.

# 7

# 6

# 5

Add end effectors to grip or scoop objects.

Pop in the cylinder pin.

String
(optional)

Test your claw!  If it’s tipping over, use string to hold it up.

Your example is done, but you aren’t… Tinker with it and keep evolving the design!

Test a lot of them – they all have different strengths and weaknesses.

Then screw in the piston.

Add your cylinder assembly to the mechanism.

# 5

# 6

# 4

# 3

# 2

# 1

Put a full strip under the screw from Step 4 and screw it in.

Half Strips

Full Strip

Make two half strips, if you don’t already have them, by cutting or snapping a full strip.

Screw the ends of the half strips together, so the screw barely sticks out the bottom.

Screw on another half strip.  It doesn’t have to match the picture exactly.

Screw a half strip to a 
full strip near the middle.

Test your claw!  Next, you are going to add hydraulics.

2x

Chopsticks Example

# 7

Add your cylinder assembly to the mechanism.

Pop in the 
cylinder pin.

# 8

Test your claw!  If it’s tipping over, use string to hold it up.

# 9

Your example is done, but you aren’t… Tinker with it and keep evolving the design!

Add end effectors to grip or scoop objects.

Test a lot of them – they all have different strengths and weaknesses.

String
(optional)

Then screw in
the piston.

Make three half strips and two quarter strips if you don’t already have them.  (This will require 2 full strips.)

3x

2x

Half Strips

Quarter Strips

# 5

# 6

# 4

# 3

# 2

# 1

Add end effectors to grip or scoop objects.

Your example is done, but you aren’t… Tinker with it; keep evolving the design!

Pop in the cylinder pin.

Screw in the piston to both quarter strips.

Use two more screws to attach the two quarter strips.

Full Strip

Test your claw!  
If it’s tipping over, 
hold it up with string (optional).

Screw the three 
half strips together.

Add your cylinder assembly.

Pincer Example

Ocean Cleanup Challenge

Design your claw to clean up the ocean!

Plastic trash is harming ocean wildlife and washing up on beaches around the world.  Use your claw to complete all three stations of the challenge (you can change your design between stations).

Learn more about seaborne trash at fws.gov/refuges/features/OceansOfTrash.html

One claw can’t do it all (at least not well), so change your claw design for each part!
The engineering design process is never done – there is no perfect design.

- You must use a claw powered by hydraulic cylinders.

- You may only use claw supplies listed on Page 1

- You have _____ minutes to complete each station of the challenge

- You have _____ minutes to change designs between each station

(rules and limits for your design)

Constraints:

Station 1: Garbage Patch

Huge amounts of trash are floating in the ocean.  Load trash into the container as fast as possible.

Criteria:

+5 points for every piece of “trash”

+1 point for every extra second left on the clock

You can use anything almost for “trash.”  
Candy, office supplies, toys, recyclables, etc.

Station 2: Load the Barge

Containers of trash must be shipped to land for processing.  Stack containers on the barge as fast as possible.

Criteria:

+5 points for each “container” on the “barge”

+1 point for every extra second left on the clock

Anything that can be stacked can be a “container.”  Snack bars, candy, blocks, etc.

Use paper or an index card for the “barge.”

GOAL 2

Claw Ball Challenge

Put a cup (goal) on each side of the room.  Each team can pick up the ball but must pass or drop the ball after walking three steps.  The first team to get 5 goals wins!

Station 3: Beach Cleanup

Pieces of trash are washing up onto beaches.  Design a claw to separate the trash from the sand without disturbing any of the wildlife.

Criteria:

Rice or sugar can be used as sand.  Candy, fishing lures, etc. can be used for trash/wildlife.

+5 points for every piece of “trash”

–10 points for every piece of wildlife

+1 point for every extra second left on the clock

GOAL 1

HORSE Challenge

Take turns picking up different objects.  If one player can pick it up, and the other players can’t, that player gets a letter towards the word “HORSE.”  The first player to spell 
“HORSE” wins!

Additional Challenges

Leave Wildlife

Remove Trash

Try making completely different designs!

Inspiration

Zip Tie

Tap or wiggle dowels into holes with teeth to secure them.

Cut slide stop with scissors.

Cutters Optional

TeacherGeek Tools unlock even more design possibilities!

Ream these two holes.

Don’t ream these holes.

Zip Tie

Ream the teeth out of holes to let dowels slide freely.

Push & Twist