Name: _______________________________________________     Set: __________     Date: __________

Perfect for Grades: 5+
Difficulty: Intermediate

Download Documents at teachergeek.com

The Lab

This lab can begin after you build the example mini wind turbine. During this lab, you 
will research rotor configurations and the effect they have on turbine voltage output. 
At the end of the lab you will devise and run your own experiment.

For use with TeacherGeek Mini Wind Turbine Activity: 1823-12 (single) or 1823-13 (10 pack)

- You’ll need the Mini Wind Turbine you built 
from the TeacherGeek Build Guide. 
https://teachergeek.com/blogs/projects/mini-wind-turbine

- Seven Skewer Sticks

| Multi-Cutter | Screwdriver | Pliers | Digital Multimeter | Alligator Clip Leads |
| --- | --- | --- | --- | --- |

Tools available at teachergeek.com

| Tape | Recycling Materials (for blades) | Fan (for testing) | Safety Goggles |
| --- | --- | --- | --- |

Name: _______________________________________________     Set: __________     Date: __________

- Write the names of the components diagramed above:

- ____________________________________________

- ____________________________________________

- ____________________________________________

- ____________________________________________

- Which of the following components is your turbine missing? 
(hub, tower, gear box, rotor, generator)

___________________________________________________________________________________________

What is RPM?
RPM = Revolutions per Minute
(the number of times something rotates in a minute)

- How many times can you spin your tubine rotor around in 30 seconds?

Revolutions in 30 seconds: ________________ RPM: _________________

- If your turbine blades rotate 400 times in two minutes, what is the

RPM of the blades?

__________________________________________________________

- What is the RPM of the second’s hand on a clock?

____________________________________________________________________________

- A fast mini turbine can spin at over 3500 RPM. How many times faster is that than 
the RPM at which you spun your turbine by hand? Show your work.

Answer: ________________________________________________________________________

Torque
Torque is a twisting force. Some turbines use a gearbox to convert torque to additional RPM. Your mini turbine does not have a gearbox, so additional torque (more torque than it takes to spin the blades) will be lost.

Note: The mini turbine will not produce enough power to light most bulbs. Measure power output with a multimeter.

- Use the following words to properly fill in the blanks. 
Use every word:    load    sun    rotational    energy    electrical    generator

The __________________________ from the wind is converted into __________________________ energy which turns the __________________________

to produce __________________________ energy. That energy is used to power a __________________________. Wind energy is created by uneven heating of the earth’s surface by the __________________________.

What can your mini turbine power?

Your mini turbine doesn’t produce enough electricity to light a bulb or run a motor (there are other TeacherGeek turbines that can).

How will you measure the power it produces?

It will produce plenty of power for a standard multi-meter to measure.

What is Voltage?

Voltage is the potential 
energy that makes the 
electrical current flow
(by pushing the electrons).
The unit of voltage is 
shown as ‘v’.

You will measure the voltage output by your turbine. Find out how below!

(Without a Load)

Set your multi-meter to measure 200-0 m volts. Connect leads from the multi-meter to the terminals on your mini turbine. The multi-meter should display a voltage output when the turbine rotor is turned. Note: Without a load, the readings on your multi-meter many not be stable. See how to test your turbine with a load on the next page.

- Voltage and RPM: Is there a correlation between RPM and Voltage? Spin your wind turbine with a finger and record your findings:

Voltage at a low RPM (spinning slow):  ____________________

Voltage at a medium RPM:	             ____________________

Voltage at a high RPM (spinning fast):  ____________________

- Describe the correlation between RPM and voltage:

____________________________________________________________________________________________________________________________________________________________________________________________________________________________________

Load 
A load is the part of an electrical circuit that “used the electricity.” The load converts the electrical energy into another form of energy.

Optional Testing:
Testing Your Turbine with a Load

The proper way to test your turbine is
to measure voltage across a load. 
Use a load if your meter measurements
jump around while you are trying to read 
them. The load can be a bulb, 2.7ohm 
resistor (SKU 1823-76), small dc motor, or 
even a pencil with both ends sharpened. 
Note: the bulb and motor with not 
light up/run, but they will still use some 
electrical energy to heat up. The same 
load should be used throughout the lab.

- Measure and graph the peak voltage output of your turbine at the distances from
 the fan shown below.

- Draw a line of best fit between your data points on the graph above.

- Describe the correlation between voltage and turbine distance from the fan:

Interpolate
to estimate values of data between two known values

_______________________________________________________________
_______________________________________________________________
_______________________________________________________________

- Using the graph above, interpolate the voltage output for the distances from the fan:

63.5cm (25in): _________________       115cm (45in): _________________

Dependent Variables

variables that change as a result of changes made to independent variables.

Independent Variables
variables you change in an experiment.

- What was the independent variable for the question 11 experiment?

____________________________________________________________________________

- What was the dependent variable for the question 11 experiment?

____________________________________________________________________________

Changing Blade Pitch

The pitch (angle) of blades can easily be changed by slightly loosening the hub screw so 
the skewer sticks can rotate, but not fall out. The screw can be retightened after all blades are adjusted to the proper angle.

Measuring Blade Pitch

The TeacherGeek protractor is the best way to easily measure blade angles.

Here’s how you use it:

The red arrows show the most common angles used on mini wind turbines

(0°, 15°, 30°, 45°, 60°, 75°, 90°).

What is the pitch of the blade shown above?

Protractor Download: http://www.teachergeek.org/protractor.pdf

________________________________________

Hypothesis
a prediction of the effects of changing one variable or another.

- Your hypothesis: How do you think changes in blade pitch will affect voltage output?

_____________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

- Measure and graph the peak voltage output of your turbine with the blades pitched to 0°, 15°, 30°, 45°, 60°, 75°and 90°. Use a TeacherGeek protractor to measure and set the blade pitch. Your turbine must be 50cm (20in) away from the fan for this experiment.

- Draw a line of best fit to connect your data points on the graph above.

- Was your hypothesis correct?  ________________________________________________________

Explain what the graph shows.

_____________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

- Is the relationship between blade pitch and voltage output linear or nonlinear?

You need to figure out what linear and nonlinear mean.

_______________________________________________________________________________________

- Use the Voltage Output /Blade Pitch Graph to calculate the ideal blade angle for the highest voltage output:

Interpolated (theoretical) blade pitch for highest voltage: ______________________________

- Adjust your turbine blades to the pitch provided for question 22. Test the wind turbine  
 with configuration used for the Voltage Output /Blade Pitch experiment (50mm away 
 from the fan). Show your teacher your turbine during testing. What is the voltage output?

Teacher Signature:  _______________________       Voltage Produced:  ____________________

- What is the difference between the actual and calculated voltage?  __________________

- What could cause the interpolated and actual voltage to be different?

______________________________________________________________________________________________________________________________________________________________________________

It is now time for you to create your own experiment and share your findings with the class.

Your experiment should test a single variable, such as:

- diameter of blades

- number of blades

- shape of blades

- mass of blades

- effect of load on voltage output

Your experiment should:

- test a hypothesis

- follow the scientific process

- document all steps

- detail findings in a conclusion

Your presentation should:

- last approximately 2 minutes

- be informative and entertaining

Example

Research the effects of blade 
diameter on voltage output. 
Create a hypothesis.

Measure voltage output for different blade lengths (rotor diameters) by progressively cutting and testing 
the blades. Graph and interpret 
the data.

Write a conclusion. Create a 
2 minute presentation documenting your experiment and findings.

document all aspects of your experiment

Experiment – 15pts

Did your experiment:

- test a hypothesis

- test a single variable

- follow the scientific process

- document all steps

- detail findings in a conclusion

Presentation Delivery – 10pts

Did your presentation:

- last approximately 2 minutes

- document all aspects of the experiment

Bonus – 2pts

Was your presentation incredibly unique, entertaining, informative and memorable?

Lab Score:      ______ /25        Experiment & Presentation Score:       _________

(Lab Score + Experiment & Presentation Score) x 2 = Overall Score

Overall Score     _________