Grades

7-12+ version available at teachergeek.com/rubberband

2-6

Download Documents at teachergeek.com/rubberband

Start by building the example racer, then turn into your own unique design.

What do you need to know, to build your racer?

Gather components to build the example racer, and then turn it into your own amazing design.

| 6 - Connector Strips | 8 - Dowels 300mm (12″) | 2 - Hole Plates | 4 - Wheels | 2 - Stretch Tires |
| --- | --- | --- | --- | --- |
| 4 - Screws #10 1″ | 4 - Nuts #10 | 1 - Slide Stop 100mm (3″) | 1 - Stop Clip | 10 - Rubber Bands |

This isn’t a kit. You’re going to really build (cut, ream, screw) your Racer. 
Here are tools you’ll need to get started. 
They can be shared by up to 4 groups at a time.

- TeacherGeek Reamer

- TeacherGeek Multi-Cutter

- Tapping Block -Optional

- Small Hammer

Tip: Save all your materials (even what you cut off). Keep them in a bag. They can be used later.

Pliers -Optional

- Philips Screwdriver

Tap or push dowels into an upside-down hole plate.

Cut four 10cm (4”) dowels.

10cm (4in)

Tap or push a hole plate

on top of the dowels.

Congratulations!

Your frame is done.

Documents at teachergeek.com/rubberband

If you are going to do the optional Sled Race, it’s now time.

Push or tap the two axles into wheels.

Cut two 13cm (5.1”) dowels. These will become axles for the wheels.

13cm (5in)

Push or tap two wheels onto the other side of the axles.

Place the axles through the frame, three holes up from the bottom.

Documents at teachergeek.com/rubberband

If you are going to do the optional Ramp Roll Lab, it’s now time.

You’re ready to roll!

Cut one 13cm (5.1”) dowel.

Set it down and let it go. Play and experiment with it.

Hook the rubber band around the stop clip. Wind up the rubber band by turning the wheels.

Cut two 1cm sections of slide stop. Use them to keep this dowel from falling out.

It takes a lot of force to snap a stop clip on. An adult may need to help.

Place the 13cm dowel through the frame, with a rubber band, as shown.

Snap on the stop

clip to one of the wheel axles.

?

13cm (5in)

Try adding more rubber bands to your racer. What happens? Do the rubber bands release their energy too fast? Can you redesign your racer so rubber bands release energy slower?

Is your stop clip spinning on the dowel? A little glue will fix it.

Use a glue stick to

keep tires from slipping

off the wheels.

1. Coat the wheel with glue.

2. Wait a few minutes for the glue to partially dry.

3. Then put the stretch tire on.

Tips

Congratulations!

Your example racer is done. Now turn it into your own design.

Traction is the friction between the wheels and the ground. It allows the vehicle to move forwards. Increase the traction (friction) between your wheels and the ground by adding tires.

Place stretch tires on the rear wheels, if you have not already. done so.

Are your wheels spinning? Maybe you need to give them more traction.

Make your racer go farther, faster, or stop on a target. 
The step-by-step instructions end here, but you’re just getting started. 
It’s time to redesign your racer for these engineering challenges.

Sprint

Redesign your racer to break a speed record or win a race.

Target

Redesign your racer to stop on a target.

Long Shot

Redesign your racer to go really far.

Download the Engineering Challenge documents at teachergeek.com/rubberband

The next pages will give you tips to help you redesign your racer.

How does your racer turn potential (stored) energy from the rubber bands into kinetic (moving) energy? Create a mechanism to release the energy over more rotations (turns) of the wheels. Adjust it for the different challenges.

Series

Parallel

Parallel or Series

Rubber bands can be connected in 
series (forming a thin, long band) or 
in parallel (forming a short, thick band).

Use Pulleys

Pulleys can be used to

change the direction of 
a rubber band or string. Dowels, that can spin, 
can be used as pulleys.

Change the Frame

Use more parts to change the frame.

Ream holes for the lever to pivot

Add a Lever

A lever can create a mechanical advantage (trading force for distance). Use it to trade force from rubber bands for more wheel revolutions (distance).

Ream the holes for the fulcrum (where the dowels go through the lever)

A string, tied to the lever, with a loop to hook on the stop clip

Rubber Bands

String wound around the axle

How does it work? The rubber bands pull the lever arm. The lever arm 
pulls the string. The string unwinds from the axle and turns the wheels.

Try Gears

Gears can be used to create a mechanical advantage (like a lever).

This gear turns five times for every time the large gear turns once.

Trouble Shooting

Will not go straight

To turn your racer into your own design, 
you are going to need more materials.  Try using…

- Extra TeacherGeek parts

- Recycled materials (food packaging, containers, bottles, cardboard, etc.)

What often makes a racer turn?

The axles are loose, or not symmetrical with the frame

The frame is not straight or square

How can you make your racer go straight?

The frame and axles are symmetrical

Slide stop can be placed on the axles to help “steer” the racer. It can keep the axles symmetrical with the racer frame, or offset (making the racer turn, or correct for a turning problem)