Learn about wind energy and power by engineering and re-engineering your very own wind-powered pumped!

TRUE

STEM

STEAM

Start here! Build your Wind Pump, evolve your design, and begin 
the Steady Wind Challenge!

-Energy & Power Lab 
 (Ages 9+)

-Steady Wind Challenge*
-Variable Wind Challenge*
-Elevation Challenge*
-Environmental Challenge*

*See Page 10

Optional Challenges

Optional Labs

Go Guide

You Are Here

Choose how you would like to complete this activity.
Download documents & videos at teachergeek.com/windpump

# Supplies

Dowels
various sizes
SKU 1821-20

Picture

Name

Qty

Hole Plates
SKU 1821-32

Block
SKU 1821-34

Cylinder
4.5 ml
SKU 1821-52

Tubing
60 cm (24 in)
SKU 1821-51

T-Connector
SKU 1821-56

Chipboard
22 cm x 5 cm
(8.5 in x 2 in)
SKU 1823-48

Project Sticks
25 cm (10 in)
SKU 1821-18

Check Valves
SKU 1821-57

Mini Hub Base
Green
SKU 1821-67

Mini Hub Cover
Green
SKU 1821-67

Mini Hub Screw
SKU 1821-67

Washers
#10 size
SKU 1821-24

Slide Stop 8 cm (3 in)
SKU 1821-49

2

1

1

2

1

2

1

1

1

10

6

1

1

1 – 5 cm (2 in)
4 – 8 cm (3 in)
4 – 10 cm (4 in)
1 – 13 cm (5 in)
2 – 15 cm (6 in)

Gear Set
SKU 1821-28

1 set
(4 gears)

Materials You Supply

- Phillips Screwdriver

- Scissors

- Duct Tape

- 2 Cups (one filled with water)

- Fan

- Stopwatch

- Recycling Bin Materials
What can you use to make turbine blades?

Modify materials to make even more creative designs with the Maker Tool Set

SKU 1823-84

Optional Tools

Have a Maker Cart?  Use Multi-Cutters 
to cut your own dowels.

PUMP Parts

These are the parts you need to build one Wind Pump, plus some extras, so you can make your own unique designs.

Included tools

TeacherGeek Reamer
SKU 1823-87

Check out the Scenario Video!  
Play it by scanning the 
QR Code or going to teachergeek.com/windpump

# Make Your Pump

Did you do it? Does your hand pump suck water from one cup and push it into the other? Awesome! You’re ready to move on to the Gears.

Mystery Parts

Tubing
30 cm (12 in)

2x

2x

1x

Tubing
30 cm (12 in)

Check Valves

Only let water (or air) through in one direction.

T-Connector

Joins 3 sections of tubing.

Tubing
15 cm (6 in)

Cylinder

Cup of Water

Empty Cup

Tinker with the Mystery Parts! 
You will need to use all of them to make your pump!

You’re going to engineer your very own hand pump!  
Later, you’ll convert it to use wind power!

Mystery Parts

Your pump must move water from one cup to the other when you push & pull the cylinder.

Can you figure out how to use these parts to build your pump?

Cut your tubing to make 
 -3x 30 cm (12 in) sections & 
 -2x 15 cm (6 in) sections.

Don’t Ream

# 4

Slide your 
largest and smallest gears 
into the frame 
so they mesh.

Finished Gears

10 cm (4 in) Dowels

15 cm (6 in) Dowel

Your pump will use gears to pump faster or with more torque.
Get your gears set up, then play with them to see how they work.

# Make Your Gears

# Gear Up

# Build Your Frame

Don’t Ream

Torque is turning force.

Your frame is done!
Time to add gears.

Push or tap another 
hole plate onto the dowels.

# 2

# 1

8 cm (3 in) 
Dowel

Hole Plate

Wiggle or tap four 8 cm (3 in) dowels into the four corners of a hole plate.

Wiggle or tap a dowel into each of your four gears.

# 3

Now you have a feel for how gears work!  Next, you’re going to use the gears to drive your pump.

40 Teeth

20 Teeth

40 Teeth

50 Teeth

Try other gear combinations

10 Teeth

20 Teeth

Gear Ratio

=

50

10

5

1

÷10

÷10

Torque Decreases

1

5

the torque

Speed Increases

5

1

= 5x the speed

Twist This Dowel

Watch & Feel This Dowel

10-Tooth Gear

50-Tooth Gear

Speed for Torque

Torque for Speed

Twist This Dowel

Watch & Feel This Dowel

10-Tooth Gear

50-Tooth Gear

Gear Ratio

=

10

50

1

5

÷10

÷10

Speed Decreases

1

5

the speed

Torque Increases

5

1

= 5x the torque

-OR-

Spin the dowels to turn the gears!

Torque is turning force.

The easier it is to twist the dowel, the more torque its gear has.

Feel the Torque

See the Speed

Watch the gears to see which one revolves faster.

Gears can trade speed for torque (and vice-versa)

# Play with Gears!

Don’t Ream

5 cm (2 in) Dowel

# 6

# 7

Use scissors 
to cut slide stop.

Test your gears!  If they don’t spin freely, loosen the slide stop until they do.  Now you’re ready to add the pump!

Secure the dowels with slide stop and a washer.

1 cm (1/2 in) Slide Stop

2 cm (1 in) Slide Stop

Washer

Mesh the 50-tooth gear and 10-tooth gear about halfway up your frame.

Cams are used on train wheels and gasoline engines.

# Geared Pump

You’re going to use your gear as a cam to convert rotational (circular) motion to linear (straight-line) motion.

# 5

# Attach the Cam

Start by connecting the cam to the frame; next, you’ll connect the cylinder.

Wiggle or tap a 5 cm (2 in) dowel into the 50-tooth gear.

1 cm (1/2 in) Slide Stop

Your geared pump is done.  It’s time to add the rotor!

Not working?

You may need to move your gears or cylinder on your frame or loosen your slide stop.

Test it out! Spin the longest dowel (10-tooth gear) to run 
your pump!

# 12

Don’t Ream

13 cm (5 in) Dowel

Block

Reamed Hole

Support

Push & Twist

(to pump)

# 8

Reamed holes have no splines (teeth) so dowels can spin freely.

Ream one of the holes in the cylinder.

Connect you pump’s cylinder to finish your geared pump.

Use the support to attach the 
cylinder.

Wiggle or tap a 13 cm (5 in) dowel into a block to make a support.

# Attach the Pump

# 11

# 10

# 9

Secure the support with slide stop.

# Ready the Rotor

Your rotor is done!  
Time to mount it on the frame.

Attach the hub cover to the hub base with the hub screw.

Hub Cover

Hub Base

Hub Screw

Complete Hub

# 15

Blade

Loosen the screw just enough to allow the blades to be pushed in (about 1 ½ turns).

# 16

Angle your blades so they spin in 
the wind.

Add all the blades and tighten your hub screw to complete the rotor.

# 17

# 13

Get six 22 cm x 5 cm (8.5 in x 2 in) pieces of chipboard.

Tape a project stick to each edge, leaving the extra on one side.

22 cm
(8.5 in)

5 cm
(2 in)

6x

# 14

# Build Your Blades

You’re going to add a rotor to harness wind energy to turn the gears that drive your pump.

Mystery Parts

This is the pump you designed on Page 2.

You may need to nudge your blades to start the pump.

Point a fan at your turbine to pump water from one cup to another!

Energy & Power Lab (Ages 14+)

Get documents and more at 
teachergeek.com/windpump

# 20

Your example wind pump is done, but you aren’t... Make it better, try a lab, or start a challenge!

Optional Lab

Tape your frame to a table.

Push the rotor onto the dowel attached to the 10-tooth gear.

# 19

# 18

# Test It Out

Blade Shape

Try all different shapes, sizes, and materials.  What can you make into a turbine blade?

Experiment with the Gears

Compound Gears

Swap the Gears

Try different gear ratios. 
What works best with your blades?

Compound gears
multiply the ratios of the other gears, giving you even more ratio options.

Blade Number

More blades can catch more wind but also creates more 
drag (air friction).

Blade Angle

Blade angle trades between speed and torque.

# Tuning Your Turbine

10

40

20

50

10

40

20

50

x

=

200

2000

=

1

10

Gear Ratio:

=

20

40

1

2

=

20

50

2

5

=

10

40

1

4

=

40

50

4

5

÷10

÷10

Experiment with the Blades

Pump water faster by harvesting as much wind energy as possible and getting the right balance between speed and torque.  Your turbine needs just enough torque to get started – any extra torque should be traded for speed.

# Steady Wind Challenge

Change setup so you pump a vertical distance of 45 cm (18 in).

- Pump 110 ml (4 oz) of water from one cup to another in the shortest time possible to win!

60 cm
(24 in)

This is the pump you designed on Page 2.

Mystery Parts

Tape the pump’s frame to a table (see Page 8).

Constraints:
(rules and limits for your design)

- Only wind can power your pump (but you can give it one small push to get it started).

- You may only use the supplies listed on Page 1.

- The fan must be 60 cm (24 in) from the blades.

- There is no limit on recycling bin materials.

# Additional Challenges

Use the Criteria & Constraints above for these challenges.

Do three trials, back-to-back, with different fan speeds.  There’s 1 minute between trials to swap gears/blades.  
The turbine 
with the 
shortest total 
time wins!

Modify your turbine to look nice in nature and have safety features to protect birds from the blades.

45 cm (18 in)

Elevation Challenge

Environmental Challenge

Variable-Wind Challenge

Engineer your pump to empty one cup into another as fast as you can!

Criteria:
(what your design must do)

Make a Kinetic Sculpture that uses the wind to make moving artwork.

## Design

## Process

## Design

## Redesign

## Test

## Evaluate

The Engineering Design Process never ends!  
There is no perfect design.

Design a Vertical Axis Turbine – they work no matter what direction the wind comes from!

Try crazy blade designs!

How else can you make your turbine?  What else can you make with the parts?

# Inspiration